SX1262 LoRa Driver Suite

Technical Documentation

Production-Grade LoRa Communication for ESP32-S3

Version 1.0.0

December 2024

Target Application: Railway Monitoring Systems

15-Minute Sensor Intervals with Real-Time Alarm Capability

# Table of Contents

1. Introduction

2. Architecture Overview

3. Quick Start Guide

4. Core Driver API

5. Radio Configuration

6. TX/RX Operations

7. Power Management

8. Optional Modules

8.1 Security Module (AES-128-CTR)

8.2 Duty Cycle Manager (ETSI Compliance)

8.3 Link Health Monitor

9. Error Handling

10. Hardware Configuration

11. Troubleshooting

12. API Reference Summary

# 1. Introduction

The SX1262 LoRa Driver Suite is a comprehensive, production-grade driver for the Semtech SX1262 LoRa transceiver, specifically designed for ESP32-S3 microcontrollers. This driver follows a "Toyota Land Cruiser" philosophy—prioritizing reliability and robustness over performance optimization.

## 1.1 Key Features

- Complete LoRa transmission and reception with hardware interrupts

- Thread-safe FreeRTOS integration with mutex protection

- Comprehensive power management (WARM/COLD sleep modes)

- Silicon errata workarounds (including critical Errata 15.2)

- Platform-agnostic architecture with clean HAL separation

- Optional security module with AES-128-CTR encryption

- ETSI duty cycle compliance manager

- Multi-node link health monitoring

## 1.2 Target Application

This driver is specifically designed for railway monitoring applications where sensors transmit data every 15 minutes with real-time alarm capability. The architecture supports bidirectional communication between sensor nodes and gateways.

## 1.3 Design Philosophy

- Reliability over performance: Every function prioritizes correct operation

- Defensive programming: Comprehensive error checking and recovery

- Modularity: Optional features don't affect core driver stability

- Testability: Extensive logging and diagnostic capabilities

# 2. Architecture Overview

The driver follows a 4-layer architecture with clear separation of concerns:

| Layer | Description |
| --- | --- |
| Application Layer | User code: sensors, gateways, application logic |
| Optional Modules | Security (AES-128-CTR), Duty Cycle, Link Health |
| Driver Layer | State machine, TX/RX operations, power management |
| SPI Protocol Layer | SX1262 command encoding, BUSY handling, diagnostics |
| HAL Layer | ESP32-S3 hardware abstraction (GPIO, SPI, timing) |

## 2.1 File Structure

| File | Purpose |
| --- | --- |
| sx1262_driver.h/cpp | Main driver: state machine, TX/RX, power management |
| sx1262_config.cpp | Radio configuration: frequency, power, modulation |
| sx1262_spi_protocol.h/cpp | SPI command layer with BUSY handling |
| sx1262_hal.h/cpp | Hardware abstraction (ESP32-S3 specific) |
| sx1262_errata.h/cpp | Silicon bug workarounds |
| sx1262_regs.h | Register and command definitions |
| config.h | Pin mappings and compile-time options |
| sx1262_security.h/cpp | Optional: AES-128-CTR encryption module |
| sx1262_duty_cycle.h/cpp | Optional: ETSI duty cycle compliance |
| sx1262_link_health.h/cpp | Optional: Multi-node health monitoring |

# 3. Quick Start Guide

Get up and running with minimal code using the simplified initialization API.

## 3.1 Minimal Transmitter Example

#include "sx1262_driver.h"

void setup() {

Serial.begin(115200);

// Initialize at 868.1 MHz, +14 dBm

if (sx1262_init_simple(868100000, 14) != SX1262_OK) {

Serial.println("Init failed!");

while(1);

}

Serial.println("Radio ready!");

}

void loop() {

uint8_t data[] = "Hello LoRa!";

sx1262_transmit(data, sizeof(data), 0, NULL);

delay(5000);

}

## 3.2 Minimal Receiver Example

void loop() {

uint8_t rx_buffer[255];

sx1262_rx_result_t result;

if (sx1262_receive(rx_buffer, 255, 10000, &result) == SX1262_OK) {

Serial.printf("Received %d bytes, RSSI: %d dBm\n",

result.payload_length, result.rssi_pkt / 2);

}

}

## 3.3 Bidirectional Communication (Sensor with ACK)

// Transmit sensor data

sx1262_transmit(sensor_data, len, 0, NULL);

// Wait for gateway ACK (500ms window)

sx1262_rx_result_t rx_result;

if (sx1262_turnaround_tx_to_rx(ack_buf, 32, 500, &rx_result) == SX1262_OK) {

// ACK received

}

// Enter sleep until next cycle

sx1262_sleep(SX1262_SLEEP_WARM);

# 4. Core Driver API

## 4.1 Initialization Functions

| Function | Description |
| --- | --- |
| sx1262_init_simple(freq, power) | Simplified init with frequency and TX power |
| sx1262_init_extended(...) | Extended init with SF and BW control |
| sx1262_driver_init() | Low-level driver initialization (no radio config) |
| sx1262_driver_deinit() | Shutdown driver and release resources |
| sx1262_driver_is_initialized() | Check if driver is ready |

## 4.2 Return Codes

| Code | Meaning |
| --- | --- |
| SX1262_OK (0) | Operation successful |
| SX1262_ERROR_INVALID_STATE (-1) | Invalid state for operation |
| SX1262_ERROR_INVALID_PARAM (-2) | Invalid parameter value |
| SX1262_ERROR_TIMEOUT (-3) | Operation timed out |
| SX1262_ERROR_SPI (-4) | SPI communication error |
| SX1262_ERROR_BUSY_TIMEOUT (-5) | BUSY pin stuck high |
| SX1262_ERROR_NOT_INITIALIZED (-6) | Driver not initialized |
| SX1262_ERROR_MUTEX (-7) | Failed to acquire mutex |
| SX1262_ERROR_HARDWARE (-9) | Hardware error (CRC, IRQ) |

## 4.3 Radio States

| State | Description |
| --- | --- |
| SX1262_STATE_UNINITIALIZED | Driver not initialized |
| SX1262_STATE_SLEEP | Low-power sleep mode |
| SX1262_STATE_STANDBY_RC | Standby with RC oscillator |
| SX1262_STATE_STANDBY_XOSC | Standby with crystal oscillator |
| SX1262_STATE_TX | Transmitting |
| SX1262_STATE_RX | Receiving |
| SX1262_STATE_ERROR | Error state (needs reset) |

# 5. Radio Configuration

## 5.1 LoRa Parameters

| Parameter | Range | Default |
| --- | --- | --- |
| Frequency | 150-960 MHz | 868.1 MHz (EU868) |
| TX Power | -9 to +22 dBm | +22 dBm |
| Spreading Factor | SF5-SF12 | SF7 (balanced) |
| Bandwidth | 7.8-500 kHz | 125 kHz (standard) |
| Coding Rate | 4/5 to 4/8 | 4/5 |
| Preamble | 6-65535 symbols | 8 symbols |
| Sync Word | 16-bit | 0x1424 (private) |

## 5.2 Configuration Structure

typedef struct {

uint32_t frequency_hz;           // RF frequency

int8_t tx_power_dbm;             // TX power (-9 to +22)

sx1262_ramp_time_t ramp_time;    // PA ramp time

sx1262_lora_sf_t spreading_factor;

sx1262_lora_bw_t bandwidth;

sx1262_lora_cr_t coding_rate;

bool low_data_rate_optimize;     // Auto-enabled for long symbols

uint16_t preamble_length;

sx1262_lora_header_t header_type;

uint8_t payload_length;

sx1262_lora_crc_t crc_type;

sx1262_lora_iq_t invert_iq;

uint16_t sync_word;

} sx1262_lora_config_t;

## 5.3 Configuration Functions

// Initialize with defaults

sx1262_config_init_defaults(&config);

// Modify as needed

config.frequency_hz = 868300000;

config.spreading_factor = SX1262_LORA_SF9;

// Apply configuration

sx1262_config_lora(&config);

# 6. TX/RX Operations

## 6.1 Transmission

sx1262_result_t sx1262_transmit(

const uint8_t* payload,    // Data to transmit (1-255 bytes)

uint8_t length,            // Payload length

uint32_t timeout_ms,       // Timeout (0 = auto from ToA)

sx1262_tx_result_t* result // Optional result structure

);

The TX result structure provides detailed transmission information:

typedef struct {

bool success;              // TX completed successfully

uint32_t tx_start_ms;      // Start timestamp

uint32_t tx_done_ms;       // Completion timestamp

uint32_t tx_duration_ms;   // Actual duration

uint32_t calculated_toa_ms;// Calculated Time-on-Air

bool timed_out;            // Timeout occurred

uint16_t irq_flags;        // IRQ status at completion

} sx1262_tx_result_t;

## 6.2 Reception

sx1262_result_t sx1262_receive(

uint8_t* payload,          // Buffer for received data

uint8_t max_length,        // Buffer size

uint32_t timeout_ms,       // RX window duration

sx1262_rx_result_t* result // Optional result structure

);

The RX result structure provides signal quality information:

typedef struct {

bool success;              // Packet received successfully

uint32_t rx_duration_ms;   // RX duration

bool timed_out;            // Timeout occurred

bool crc_error;            // CRC error detected

uint8_t payload_length;    // Received payload length

int16_t rssi_pkt;          // RSSI (divide by 2 for dBm)

int8_t snr_pkt;            // SNR in dB

} sx1262_rx_result_t;

## 6.3 TX/RX Turnaround Functions

For bidirectional communication, use the turnaround functions:

// After TX, wait for response

sx1262_turnaround_tx_to_rx(rx_buf, 255, 500, &rx_result);

// After RX, send response

sx1262_turnaround_rx_to_tx(tx_buf, len, 0, &tx_result);

# 7. Power Management

## 7.1 Sleep Modes

The SX1262 supports two sleep modes with different power/wake tradeoffs:

| Mode | Current | Wake Time | Config |
| --- | --- | --- | --- |
| WARM | ~600 nA | ~340 µs | Retained |
| COLD | ~160 nA | ~3.5 ms | Lost (auto-restored) |

## 7.2 Sleep/Wake API

// Enter sleep mode

sx1262_sleep(SX1262_SLEEP_WARM);  // Config retained

sx1262_sleep(SX1262_SLEEP_COLD);  // Lowest power

// Wake from sleep

sx1262_wake();  // Auto-restores config for COLD sleep

// Check sleep state

if (sx1262_is_sleeping()) { ... }

// Get power statistics

uint32_t total_sleep_ms, wake_count;

sx1262_get_power_stats(&total_sleep_ms, &wake_count);

## 7.3 Typical Sensor Duty Cycle

void loop() {

// Wake from sleep

if (sx1262_is_sleeping()) {

sx1262_wake();

}

// Read sensor and transmit

uint8_t data[16];

read_sensor(data);

sx1262_transmit(data, sizeof(data), 0, NULL);

// Wait for ACK

sx1262_turnaround_tx_to_rx(ack_buf, 32, 500, NULL);

// Enter sleep for 15 minutes

sx1262_sleep(SX1262_SLEEP_WARM);

esp_deep_sleep(15 * 60 * 1000000ULL);

}

# 8. Optional Modules

The driver suite includes three optional modules that can be enabled independently without modifying core driver files.

## 8.1 Security Module (AES-128-CTR)

Provides payload encryption with replay protection for secure communication.

### Features

- AES-128-CTR stream cipher encryption

- 4-byte counter for replay protection

- Automatic counter persistence via callbacks

- Platform-agnostic crypto HAL

### Usage

// Initialize with 128-bit key

uint8_t key[16] = { 0x00, 0x11, 0x22, ... };

sx1262_security_context_t sec_ctx;

sx1262_security_init(&sec_ctx, key, 0);

// Encrypt before transmission

uint8_t buffer[64];

memcpy(buffer, payload, payload_len);

uint8_t secure_len;

sx1262_secure_pack(&sec_ctx, buffer, payload_len, &secure_len);

sx1262_transmit(buffer, secure_len, 0, NULL);

// Decrypt after reception

uint8_t decrypted_len;

sx1262_secure_unpack(&sec_ctx, rx_buffer, rx_len, &decrypted_len);

### Counter Persistence

For security to survive reboots, the TX counter must be saved to non-volatile storage:

// With automatic storage callbacks (recommended)

sx1262_security_storage_t storage = {

.save = my_nvs_save,

.load = my_nvs_load,

.save_interval = 0,     // Save every TX

.safety_margin = 100    // Skip 100 on load

};

sx1262_security_init_with_storage(&sec_ctx, key, &storage, 0);

## 8.2 Duty Cycle Manager (ETSI Compliance)

Tracks transmission airtime and enforces regulatory duty cycle limits.

### ETSI Sub-bands

| Sub-band | Frequency | Duty Cycle |
| --- | --- | --- |
| g1 | 868.0-868.6 MHz | 1% (36 sec/hour) |
| g2 | 868.7-869.2 MHz | 0.1% (3.6 sec/hour) |
| g3 | 869.4-869.65 MHz | 10% (360 sec/hour) |

### Usage

// Initialize for ETSI g1 (1% duty cycle)

sx1262_dc_init(SX1262_DC_REGION_ETSI_G1);

// Before each transmission

uint32_t wait_ms;

uint32_t toa = sx1262_get_time_on_air_ms(&config, payload_len);

if (sx1262_dc_can_transmit(toa, &wait_ms)) {

sx1262_transmit(payload, len, 0, &result);

sx1262_dc_record_transmission(result.tx_duration_ms);

} else {

Serial.printf("Must wait %lu ms\n", wait_ms);

}

## 8.3 Link Health Monitor

Tracks communication quality with remote nodes for gateway applications.

### Alarm Types

- SILENT: Node hasn't transmitted in expected window

- IRREGULAR: Transmission timing varies significantly

- WEAK_SIGNAL: RSSI below threshold

- RECOVERED: Previously alarmed node recovered

### Usage

// Initialize with callback

void on_alarm(const sx1262_link_alarm_t* alarm) {

Serial.printf("Node %d: %s\n", alarm->node_id,

sx1262_link_alarm_type_to_string(alarm->type));

}

sx1262_link_health_init(20, on_alarm);

// Register expected nodes (15-min interval = 900000 ms)

sx1262_link_health_register_node(0x01, 900000);

sx1262_link_health_register_node(0x02, 900000);

// Record receptions

sx1262_link_health_record_rx(node_id, rssi, snr);

// Periodic check (in main loop)

sx1262_link_health_check_timeouts();

# 9. Error Handling

## 9.1 Error Checking Pattern

sx1262_result_t res = sx1262_transmit(data, len, 0, NULL);

if (res != SX1262_OK) {

Serial.printf("TX failed: %s\n", sx1262_error_to_string(res));

// Get detailed error info

sx1262_result_t err;

const char* func;

uint32_t timestamp;

sx1262_get_last_error(&err, &func, &timestamp);

}

## 9.2 Watchdog Protection

The driver includes a watchdog that triggers emergency reset after consecutive failures:

// Enable watchdog with 3 failure threshold

sx1262_watchdog_init(3);

sx1262_watchdog_enable(true);

// Feed watchdog after successful operations

if (res == SX1262_OK) {

sx1262_watchdog_feed();

}

## 9.3 Emergency Reset

If the radio enters an unrecoverable state, the driver automatically performs hardware reset and re-initialization.

# 10. Hardware Configuration

## 10.1 Default Pin Mapping (ESP32-S3)

| Function | GPIO | Notes |
| --- | --- | --- |
| NRESET | 16 | Hardware reset control |
| BUSY | 5 | Ready/busy status indicator |
| DIO1 | 2 | Interrupt output (TX/RX done) |
| CS (NSS) | 10 | SPI chip select |
| MOSI | 11 | SPI Master Out |
| MISO | 13 | SPI Master In |
| SCLK | 12 | SPI Clock |

## 10.2 Modifying Pin Configuration

Edit config.h to change pin assignments:

#define PIN_SX1262_NRESET   16

#define PIN_SX1262_BUSY     5

#define PIN_SX1262_DIO1     2

#define PIN_SX1262_CS       10

## 10.3 SPI Configuration

#define SPI_FREQUENCY_HZ    10000000  // 10 MHz

#define PIN_SPI_MOSI        11

#define PIN_SPI_MISO        13

#define PIN_SPI_SCLK        12

# 11. Troubleshooting

## 11.1 Common Issues

### BUSY Timeout on Init

- Check wiring to BUSY pin

- Verify power supply (3.3V, >200mA capability)

- Try hardware reset before init

### TX Timeout

- Verify antenna connection

- Check frequency is within valid range

- Increase timeout value

### No RX Packets

- Verify TX and RX use same frequency/SF/BW

- Check sync word matches on both ends

- Ensure CRC setting matches

### Configuration Not Retained After Sleep

- Use WARM sleep for config retention

- COLD sleep requires full reconfig (handled automatically)

## 11.2 Debug Logging

Enable verbose logging in config.h:

#define DEBUG_ENABLED       1

#define DEBUG_VERBOSE_SPI   1

#define DEBUG_VERBOSE_STATE 1

## 11.3 Silicon Errata

The driver includes workarounds for known SX1262 silicon bugs:

- Errata 15.2: IRQ config cleared after SetRx - automatically re-applied

- Sensitivity degradation fix applied after wake

- PA clamp voltage optimization for high power

# 12. API Reference Summary

## 12.1 Core Functions

// Initialization

sx1262_init_simple(freq_hz, power_dbm)

sx1262_driver_init()

sx1262_driver_deinit()

// TX/RX Operations

sx1262_transmit(payload, length, timeout_ms, result)

sx1262_receive(buffer, max_len, timeout_ms, result)

sx1262_turnaround_tx_to_rx(...)

sx1262_turnaround_rx_to_tx(...)

// Power Management

sx1262_sleep(mode)

sx1262_wake()

sx1262_is_sleeping()

sx1262_get_power_stats(...)

// Configuration

sx1262_config_lora(config)

sx1262_set_frequency(freq_hz)

sx1262_set_tx_power(power_dbm, ramp_time)

sx1262_get_time_on_air_ms(config, payload_len)

## 12.2 Security Module

sx1262_security_init(ctx, key, initial_counter)

sx1262_security_init_with_storage(ctx, key, storage, fallback)

sx1262_secure_pack(ctx, buffer, len, out_len)

sx1262_secure_unpack(ctx, buffer, len, out_len)

sx1262_security_force_save(ctx)

## 12.3 Duty Cycle Manager

sx1262_dc_init(region)

sx1262_dc_can_transmit(airtime_ms, wait_ms)

sx1262_dc_record_transmission(airtime_ms)

sx1262_dc_get_budget_remaining_ms()

## 12.4 Link Health Monitor

sx1262_link_health_init(max_nodes, callback)

sx1262_link_health_register_node(node_id, interval_ms)

sx1262_link_health_record_rx(node_id, rssi, snr)

sx1262_link_health_check_timeouts()

— End of Documentation —