# SX1262 Driver Suite

Complete Function Reference

Version 1.0.0 | December 2024

# Table of Contents

- Initialization Functions

- Configuration Functions

- TX/RX Operations

- Power Management

- State Management

- Duty Cycle Management (Optional)

- Security Module (Optional)

- Link Health Monitor (Optional)

- Diagnostics & Statistics

- Low-Level SPI Functions

# 1. Initialization Functions

## sx1262_init_simple()

Initialize the driver with minimal parameters using sensible defaults.

Prototype:

sx1262_result_t sx1262_init_simple(uint32_t frequency_hz, int8_t tx_power_dbm);

Parameters:

frequency_hz: RF frequency in Hz (e.g., 868100000 for 868.1 MHz)

tx_power_dbm: Transmit power in dBm (-9 to +22)

Returns:

SX1262_OK on success, error code on failure

Example:

if (sx1262_init_simple(868100000, 14) == SX1262_OK) {

Serial.println("Radio initialized!");

}

## sx1262_init_extended()

Initialize with control over spreading factor and bandwidth.

Prototype:

sx1262_result_t sx1262_init_extended(uint32_t frequency_hz, int8_t tx_power_dbm,

sx1262_lora_sf_t sf, sx1262_lora_bw_t bw);

Example:

sx1262_init_extended(868100000, 14, SX1262_LORA_SF7, SX1262_LORA_BW_125);

## sx1262_self_test()

Perform hardware self-test to verify wiring before full initialization.

Prototype:

sx1262_result_t sx1262_self_test(sx1262_self_test_result_t* result);

Example:

sx1262_self_test_result_t test;

if (sx1262_self_test(&test) != SX1262_OK) {

Serial.printf("FAILED: %s\n", test.failure_reason);

}

## sx1262_driver_deinit()

Deinitialize driver and release all resources.

sx1262_result_t sx1262_driver_deinit(void);

# 2. Configuration Functions

## sx1262_set_frequency()

Set the RF frequency. Valid range: 150 MHz to 960 MHz.

sx1262_result_t sx1262_set_frequency(uint32_t frequency_hz);

Example:

sx1262_set_frequency(915000000);  // Switch to 915 MHz

## sx1262_set_tx_power()

Set transmit power and PA ramp time.

sx1262_result_t sx1262_set_tx_power(int8_t power_dbm, sx1262_ramp_time_t ramp_time);

Example:

sx1262_set_tx_power(22, SX1262_RAMP_40U);  // Max power, 40us ramp

## sx1262_set_modulation_params()

Configure LoRa modulation: SF, BW, CR, LDRO.

sx1262_result_t sx1262_set_modulation_params(sx1262_lora_sf_t sf,

sx1262_lora_bw_t bw, sx1262_lora_cr_t cr, bool ldro);

Example:

sx1262_set_modulation_params(SX1262_LORA_SF10, SX1262_LORA_BW_125,

SX1262_LORA_CR_4_5, true);

## sx1262_set_sync_word()

Set the LoRa sync word for network isolation.

sx1262_result_t sx1262_set_sync_word(uint16_t sync_word);

Example:

sx1262_set_sync_word(0x3444);  // Private network

sx1262_set_sync_word(0x1424);  // Public LoRaWAN

## sx1262_get_time_on_air_ms()

Calculate expected transmission time for a given payload length.

uint32_t sx1262_get_time_on_air_ms(const sx1262_lora_config_t* config, uint8_t payload_len);

Example:

uint32_t toa = sx1262_get_time_on_air_ms(NULL, 20);  // NULL uses current config

Serial.printf("Expected TX time: %lu ms\n", toa);

## sx1262_print_config()

Print current radio configuration to serial output.

void sx1262_print_config(void);

# 3. TX/RX Operations

## sx1262_transmit()

Transmit a LoRa packet. Blocking until TX completes or times out.

sx1262_result_t sx1262_transmit(const uint8_t* payload, uint8_t length,

uint32_t timeout_ms, sx1262_tx_result_t* result);

Parameters:

payload: Data buffer to transmit (1-255 bytes)

length: Number of bytes to send

timeout_ms: TX timeout (0 = auto-calculate from ToA)

result: Optional pointer to TX result structure

Example:

uint8_t data[] = "Hello LoRa!";

sx1262_tx_result_t result;

if (sx1262_transmit(data, sizeof(data), 0, &result) == SX1262_OK) {

Serial.printf("TX done in %lu ms\n", result.tx_duration_ms);

}

## sx1262_receive()

Receive a LoRa packet. Blocking until packet received or timeout.

sx1262_result_t sx1262_receive(uint8_t* payload, uint8_t max_length,

uint32_t timeout_ms, sx1262_rx_result_t* result);

Result structure contains:

payload_length: Received data length

rssi_pkt: Signal strength (dBm)

snr_pkt: Signal-to-noise ratio (dB)

Example:

uint8_t rx_buf[255];

sx1262_rx_result_t result;

if (sx1262_receive(rx_buf, 255, 5000, &result) == SX1262_OK) {

Serial.printf("Got %d bytes, RSSI: %d\n",

result.payload_length, result.rssi_pkt);

}

## sx1262_turnaround_tx_to_rx()

Fast transition from TX to RX mode (for waiting for ACK after transmit).

sx1262_result_t sx1262_turnaround_tx_to_rx(uint8_t* rx_payload, uint8_t max_length,

uint32_t timeout_ms, sx1262_rx_result_t* result);

Example:

// After TX, wait for ACK

sx1262_transmit(data, len, 0, NULL);

if (sx1262_turnaround_tx_to_rx(ack_buf, 32, 2000, &rx) == SX1262_OK) {

Serial.println("ACK received!");

}

## sx1262_turnaround_rx_to_tx()

Fast transition from RX to TX mode (for sending ACK after receive).

sx1262_result_t sx1262_turnaround_rx_to_tx(const uint8_t* tx_payload, uint8_t length,

uint32_t timeout_ms, sx1262_tx_result_t* result);

# 4. Power Management

## sx1262_sleep()

Enter low-power sleep mode.

sx1262_result_t sx1262_sleep(sx1262_sleep_mode_t mode);

Sleep modes:

SX1262_SLEEP_WARM: 600 nA, 340 µs wake, config retained

SX1262_SLEEP_COLD: 160 nA, 3.5 ms wake, config lost

Example:

sx1262_sleep(SX1262_SLEEP_WARM);  // Sleep until next TX

## sx1262_wake()

Wake from sleep mode and return to STANDBY_RC.

sx1262_result_t sx1262_wake(void);

Example:

sx1262_wake();

// After cold start, must reconfigure:

if (!sx1262_is_config_valid()) {

sx1262_config_lora(&saved_config);

}

## sx1262_is_sleeping()

Check if radio is currently in sleep mode.

bool sx1262_is_sleeping(void);

Example:

if (sx1262_is_sleeping()) sx1262_wake();

## sx1262_get_power_stats()

Get cumulative sleep statistics for power analysis.

void sx1262_get_power_stats(uint32_t* total_sleep_ms, uint32_t* wake_count);

# 5. State Management

## sx1262_get_state()

Get current radio state.

sx1262_state_t sx1262_get_state(void);

States:

SX1262_STATE_UNINITIALIZED, SX1262_STATE_SLEEP,

SX1262_STATE_STANDBY_RC, SX1262_STATE_TX, SX1262_STATE_RX

## sx1262_reset()

Perform hardware reset (toggle NRESET pin).

sx1262_result_t sx1262_reset(void);

## sx1262_prepare_for_tx() / sx1262_prepare_for_rx()

Ensure radio is in clean state before TX/RX operations.

sx1262_result_t sx1262_prepare_for_tx(void);

sx1262_result_t sx1262_prepare_for_rx(void);

# 6. Duty Cycle Management (Optional Module)

Include: sx1262_duty_cycle.h

## sx1262_dc_init()

Initialize duty cycle manager with regional preset.

sx1262_dc_result_t sx1262_dc_init(sx1262_dc_region_t region);

Regions:

SX1262_DC_REGION_ETSI_G1: 1% duty cycle (868 MHz)

SX1262_DC_REGION_ETSI_G3: 10% duty cycle

Example:

sx1262_dc_init(SX1262_DC_REGION_ETSI_G1);

## sx1262_dc_can_transmit()

Check if transmission is allowed under duty cycle limits.

bool sx1262_dc_can_transmit(uint32_t airtime_ms, uint32_t* out_wait_ms);

Example:

uint32_t wait_ms;

uint32_t toa = sx1262_get_time_on_air_ms(NULL, payload_len);

if (sx1262_dc_can_transmit(toa, &wait_ms)) {

sx1262_transmit(payload, len, 0, &result);

sx1262_dc_record_transmission(result.tx_duration_ms);

} else {

Serial.printf("Must wait %lu ms\n", wait_ms);

}

## sx1262_dc_record_transmission()

Record a completed transmission (call AFTER tx_transmit).

sx1262_dc_result_t sx1262_dc_record_transmission(uint32_t airtime_ms);

## sx1262_dc_get_budget_remaining_ms()

Get remaining airtime budget in current window.

uint32_t sx1262_dc_get_budget_remaining_ms(void);

# 7. Security Module (Optional)

Include: sx1262_security.h

## sx1262_security_init()

Initialize security context with AES-128 key.

sx1262_security_result_t sx1262_security_init(sx1262_security_context_t* ctx,

const uint8_t* key, uint32_t initial_counter);

Example:

sx1262_security_context_t sec;

uint8_t key[16] = {0x2B, 0x7E, 0x15, ...};  // Your 128-bit key

sx1262_security_init(&sec, key, 0);

## sx1262_secure_pack()

Encrypt payload for transmission (in-place, adds 4-byte counter).

sx1262_security_result_t sx1262_secure_pack(sx1262_security_context_t* ctx,

uint8_t* buffer, uint8_t payload_len, uint8_t* out_total_len);

Example:

uint8_t buffer[64];

memcpy(buffer, payload, payload_len);

uint8_t secure_len;

sx1262_secure_pack(&sec, buffer, payload_len, &secure_len);

sx1262_transmit(buffer, secure_len, 0, NULL);

## sx1262_secure_unpack()

Decrypt received payload with replay protection.

sx1262_security_result_t sx1262_secure_unpack(sx1262_security_context_t* ctx,

uint8_t* buffer, uint8_t total_len, uint8_t* out_payload_len);

# 8. Link Health Monitor (Optional)

Include: sx1262_link_health.h

## sx1262_link_health_init()

Initialize link health monitor with alarm callback.

sx1262_link_result_t sx1262_link_health_init(uint8_t max_nodes,

sx1262_link_alarm_callback_t callback);

Example:

void on_alarm(const sx1262_link_alarm_t* alarm) {

Serial.printf("ALARM: Node %d silent!\n", alarm->node_id);

}

sx1262_link_health_init(20, on_alarm);

## sx1262_link_health_register_node()

Register a node to track with expected TX interval.

sx1262_link_result_t sx1262_link_health_register_node(uint8_t node_id,

uint32_t expected_interval_ms);

Example:

sx1262_link_health_register_node(0x01, 900000);  // 15-minute interval

## sx1262_link_health_record_rx()

Record successful packet reception from a node.

sx1262_link_result_t sx1262_link_health_record_rx(uint8_t node_id,

int16_t rssi, int8_t snr);

## sx1262_link_health_check_timeouts()

Check for silent nodes and trigger alarms. Call periodically.

uint8_t sx1262_link_health_check_timeouts(void);  // Returns count of alarmed nodes

# 9. Diagnostics & Statistics

## sx1262_get_statistics()

Get driver statistics including state transitions and errors.

void sx1262_get_statistics(sx1262_statistics_t* stats);

## sx1262_print_status()

Print current driver status to serial.

void sx1262_print_status(void);

## sx1262_get_last_error()

Get information about the last error that occurred.

void sx1262_get_last_error(sx1262_result_t* error, const char** function,

uint32_t* timestamp_ms);

## sx1262_error_to_string()

Convert error code to human-readable string.

const char* sx1262_error_to_string(sx1262_result_t error);

# 10. Low-Level SPI Functions

Include: sx1262_spi_protocol.h (advanced use only)

## spi_get_status()

Read chip status directly via SPI.

spi_result_t spi_get_status(uint8_t* status);

## spi_get_irq_status()

Read current IRQ flags.

spi_result_t spi_get_irq_status(uint16_t* irq_status);

## spi_clear_irq_status()

Clear specified IRQ flags.

spi_result_t spi_clear_irq_status(uint16_t irq_mask);

## spi_read_register() / spi_write_register()

Direct register access for advanced debugging.

spi_result_t spi_read_register(uint16_t addr, uint8_t* data, uint16_t len);

spi_result_t spi_write_register(uint16_t addr, const uint8_t* data, uint16_t len);

# Quick Reference: Return Codes

| Code | Description |
| --- | --- |
| SX1262_OK | Success (0) |
| SX1262_ERROR_INVALID_STATE | Invalid state transition attempted |
| SX1262_ERROR_INVALID_PARAM | Invalid parameter passed |
| SX1262_ERROR_TIMEOUT | Operation timed out |
| SX1262_ERROR_SPI | SPI communication error |
| SX1262_ERROR_BUSY_TIMEOUT | BUSY pin stuck high |
| SX1262_ERROR_NOT_INITIALIZED | Driver not initialized |
| SX1262_ERROR_HARDWARE | Hardware error (CRC, chip failure) |

# Quick Reference: LoRa Parameters

| Spreading Factor | Bandwidth | Coding Rate |
| --- | --- | --- |
| SX1262_LORA_SF5 (5) | SX1262_LORA_BW_7P8 | SX1262_LORA_CR_4_5 |
| SX1262_LORA_SF6 (6) | SX1262_LORA_BW_15P6 | SX1262_LORA_CR_4_6 |
| SX1262_LORA_SF7 (7) ★ | SX1262_LORA_BW_31P25 | SX1262_LORA_CR_4_7 |
| SX1262_LORA_SF8 (8) | SX1262_LORA_BW_62P5 | SX1262_LORA_CR_4_8 |
| SX1262_LORA_SF9 (9) | SX1262_LORA_BW_125 ★ |  |
| SX1262_LORA_SF10 (10) | SX1262_LORA_BW_250 |  |
| SX1262_LORA_SF11 (11) | SX1262_LORA_BW_500 |  |
| SX1262_LORA_SF12 (12) |  |  |

★ = Recommended default